Practical 1

1) Write a 8051 C program to sending values 00-FF to port P1

#include <reg51.h>

void main(void)

{

unsigned char z;

for(z=0;z<=255;z++)

P1=z;

}

Practical 2

2.Sending characters “YES” TO UART with baudrate 9600 using serial communication

#include<reg51.h>

void send(char x);

void main(void)

{

TMOD=0x20;

TH1=0xFD;

SCON=0x50;

TR1=1;

send('Y');

send('E');

send('S');

send('\r');

while(1);

}

void send(char x)

{

SBUF=x;

while(TI==0);

TI=0;

}

}

Practical 3

3. Configuring Timer Counter Register(TCON) of 8051 microcontroller and generate time delay to enable interrupt(Toggling two values(10101010 and 01010101) in port P1 with the given time delay)

#include<reg51.h>

void ToDelay(void);

void main(void)

{

while(1)

{

P1=0x55;

ToDelay();

P1=0xAA;

ToDelay();

}

}

void ToDelay()

{

TMOD=0x01;

TL0=0x00;

TH0=0x35;

TR0=1;

while(TF0==0);

TR0=0;

TF0=0;

}

Practical 4

4. Generating Square wave In D/A converter(logic analyser window)

#include<reg51.h>

void delay();

void main()

{

while(1){

P1=0xff;

delay();

P1=0x00;

delay();

}

}

void delay(){

unsigned int i,j;

for(i=0;i<100;i++)

for(j=0;j<1275;j++);

}

Practical 5

5. Simulating 8 bit binary counter

include<reg51.h>

void delay(int time);

void main()

{

P1 = 00000000;

while(1)

{

P1++;

delay(100);

}

}

void delay(int time)

{

int i,j;

for(i=0;i<=time;i++)

for(j=0;j<=23;j++);

}

Practical 6

6. Generating triangular wave in D/A converter for given frequency

#include<reg51.h>

void main()

{

P1=0x00;

while(1)

{

do

{

P1+=0x05;

}

while(P1<0xFF);

do

{

P1-=0x05;

}

while(P1>0x00);

}

}

Tkinkercad

Generating pattern using three LED’S

CODE:

void setup()

{

pinMode(13, OUTPUT);

pinMode(8, OUTPUT);

pinMode(7, OUTPUT);

}

void loop()

{

digitalWrite(13, HIGH);

delay(1000);

digitalWrite(13, LOW);

delay(1000);

digitalWrite(8, HIGH);

delay(500);

digitalWrite(8, LOW);

delay(500);

digitalWrite(8, HIGH);

delay(500);

digitalWrite(8, LOW);

delay(500);

for(int i = 0;i <3;i = i+1)

{

digitalWrite(7, HIGH);

delay(500);

digitalWrite(7,LOW);

delay(500);

}

}

FADING A LIGHT

Code:

int led = 9;

int brightness = 0;

int fadeAmount = 5;

void setup()

{

pinMode(led, OUTPUT);

}

void loop()

{

analogWrite(led, brightness);

brightness = brightness + fadeAmount;

if (brightness <= 0 || brightness >= 255)

{

fadeAmount = -fadeAmount;

}

delay(30);

}

Output:

Home Automation 1

Interfacing LCD (Output) with Arduino

//C++ Code

#include<LiquidCrystal.h>

LiquidCrystal lcd(12,11,5,4,3,2);

void setup()

{

lcd.begin(16,2);

}

void loop()

{

lcd.setCursor(0,0);

lcd.print("Welcome to");

lcd.setCursor(2,1);

lcd.print(" SIES College");

}

PIR Sensor with Arduino

CODE:

OUTPUT:

Interfacing Ultrasonic sensor with Arduino

CODE:

const int TRIG_PIN = 13; //Arduino pin connected to Ultrasonic Sensor's TRIG pin

const int ECHO_PIN = 12; //Arduino pin connected to Ultrasonic Sensor's TRIG pin

const int LED_PIN = 11; //Arduino pin Connected to Ultrasonic LED's pin

const int DISTANCE_THRESHOLD = 50; //centimeters

//variables wil change:

float duration_us, distance_cm;

void setup() {

Serial.begin(9600); //initialize serial port 9600 - Baud rate

pinMode(TRIG_PIN, OUTPUT); //set arduino pin to output mode

pinMode(ECHO_PIN, INPUT); //set arduino pin to output mode

pinMode(LED_PIN, OUTPUT); //set arduino pin to output mode

}

void loop()

{

//generate 10-microsecound pulse to TRIG pin

digitalWrite(TRIG_PIN,HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN,LOW);

//measure duration of pulse from ECHO pin

duration_us = pulseIn(ECHO_PIN, HIGH); //capture the lenth of the pulse and if no pulse capture 0

distance_cm = 0.017 * duration_us;

if(distance_cm < DISTANCE_THRESHOLD)

digitalWrite(LED_PIN, HIGH); // turn on LED

else

digitalWrite(LED_PIN, LOW); // turn off LED

//print the value of serial monitor

Serial.print("distance: ");

Serial.print(distance_cm);

Serial.println(" cm");

delay(500);

}

OUTPUT: